AN EXTENDED KALMAN FILTER APPLICATION IN THE UNDERWATER VEHICLE POSITIONING TASK

Aleksey Kabanov*, Vadim Kramar

Department of Informatics and Control in Technical Systems, Sevastopol State University, 299053 Sevastopol, Russia

*Author to whom correspondence should be addressed.

A B S T R A C T

This paper proposes a sensor fusion scheme for using an extended Kalman filter in the underwater vehicle positioning task by means of communication devices (buoys) giving location using a slant range mechanism, inertial sensors, a Doppler log, and a pressure sensor. The parameter estimation methods for all navigation system components are described. The results of simulation modeling with corresponding quality metrics are presented.

1. Introduction

Autonomous underwater vehicles (AUVs) in general are robots that are capable of moving underwater independently without operator assistance. Until recently, AUVs were used only in limited applications. With the development of data processing technology and high-efficiency power supplies, AUVs have evolved and been used more frequently.

Generally, AUVs have a high level of autonomy, flexibility, and remote navigation capabilities for marine resource exploration, underwater detection, data collection, and other aspects [1]. With the development of air and land navigation technology, underwater navigation technology has also evolved significantly [2,3,4]. However, due to the specifics of the underwater environment, there is still a gap between the accuracy of navigation and positioning of underwater vehicles compared to airborne and land-based vehicles, and at the moment underwater navigation has become a major stumbling block in the field of marine research [5].

In summary, even with more sensors, there are still many limitations and disadvantages to independent AUV operation underwater. However, AUVs are able to use their navigation sensors to obtain relevant measurement information for subsequent integration with underwater acoustic communication technologies. The ease and flexibility of using such systems has led to increased researcher attention to underwater AUV positioning [6,7].

The main navigation and positioning problem is state estimation, and accurate observational data collection. Most existing AUV positioning methods include improved Kalman filter (KF) based navigation algorithms combined with measurement data from underwater navigation sensors. The combined AUV localization system model is often nonlinear, so an extended Kalman filter (EKF) is usually used to estimate the state [8,9,10].

Currently, the use of hydroacoustic communication devices to obtain relative observations is the most effective and reliable measurement method. However, AUVs are used in complex marine environments where the measurement sensors can be affected by unfavorable conditions, leading to unknown errors of the measurement system. This inevitably leads to decreased accuracy and stability of the filtering procedure.

Among the existing solutions for the positioning problem, the work [11] stands out. There was considered a system of sensors data fusion, among which the values of slant range from three hydroacoustic buoys located on the surface. The similar problem was presented in [12], except that four buoys are already involved. In the presented sources it is possible to allocate essential lacks: absence of tuning parameters for the filter and sensors fusion methodology. Moreover, quality metrics for filter errors are not given, which makes it difficult to determine the accuracy of the state recovery algorithms.

To solve the above problems, it is necessary to develop an algorithm for filtering with EKF, taking into account the noise parameters of the sensors. In the present work it will be accepted to use two buoys on the surface to provide positioning of the submersible when complexing information with inertial navigation sensors, Doppler logger and pressure sensor.

The hydroacoustic transceiver arrangement was based on [11], except for the existence of only two buoys. The studies [13,14] were used as a basis for mathematical modeling of the navigation system objects dynamics (ANPA and buoys). Hydrodynamic parameters were determined using tabular data [15,16], and SDC-form [17] was used to represent all equations of dynamics and kinematics. The expressions presented in [18] were used to simulate the measurements of each sensor in the navigation system. The extended Kalman filter for state filtering was formed according to the general methodology presented in a number of sources [8-12].

The present paper has the following structure: the second section is devoted to the used methods and means for the positioning task, the third section describes the components of the proposed navigation system, and the fourth section is devoted to the simulation modeling. The fifth section contains conclusions on the work done and further research directions.

2. Materials and methods

2.1. AUV navigation systems

In contrast to airborne or ground-based drones, AUVs are dealing with a uniquely difficult navigational problem due to the lack of high-precision satellite navigation underwater. Of course, for remotely piloted vehicles, additional navigation information (position, speed) may be sent to the vehicle via fiber optic cable. However, for unmanned submersibles without cable communication, this is almost impossible to be implemented in practice [19].

There are three main methods of AUV navigation in the literature: dead reckoning and inertial navigation, acoustic navigation and geophysical navigation methods [20].

The first method is based primarily on the inertial navigation equipment, which has become financially affordable, especially after the creation of microelectromechanical systems (MEMS).

Since measurement errors of inertial navigation equipment are monotonically increasing and unlimited, other aids (e.g., differential global positioning system for position estimation, Doppler velocity log or correlated speed log for velocity estimation; pressure sensors for depth estimation, etc.) must be integrated to improve positioning system accuracy [18].

Acoustic navigation is based on using the AUV transponder's acoustic signals to determine its position. The most common methods are the long baseline, which uses at least two widely separated transponders mounted usually on the seafloor; and the ultra-short baseline, which uses GPS-calibrated transponders on an accompanying surface vessel. Both methods have limited range (about 10 km for individual LBLs, about 4 km in deep water, while less than 0.5 km in shallow water for USBL networks). Because LBL requires the installation of beacons, its applicability is limited to missions performed in stationary locations (e.g., harbor defense). In addition, beacon installation and maintenance are complicated and expensive operations. USBL may not be applicable in some military applications because of tactical limitations, as it requires an accompanying vessel [21].

As can be seen in Fig. 1, it is necessary to provide measurements fusion of different sensors in order to estimate AUV position together with errors.

GYROSCOPE

ACCELEROMETER

PRESSURE SENSOR

DOPPLER VELOCITY LOG

GPS

EXTENDED KALMAN FILTER

x0

P0

Q

R

x

y

pb1, pb2

uDVL, vDVL

z

νIMU

η2GYRO

Figure 1 – Fusion and filtering scheme for sensor readings

The most commonly used method of obtaining absolute position information underwater is through the buoys. These buoys are in known locations and the AUV receives the range and/or azimuth to several of them and then calculates its position through trilateration or triangulation. Based on the location of the transceivers, three different basic systems can be distinguished: Long Baseline System (LBL), Short Baseline System (SBL) and Ultra Short Baseline System (USBL) [22,23].

A typical configuration for a standard long baseline is shown in Figure 2(a). Two or more buoys are deployed around the perimeter of the area in which the AUV will operate. These buoys are anchored and float on the surface or, especially in deeper waters, several meters above the seafloor. Each unit receives acoustic requested pings from a common receiving channel. After receiving a request ping from the AUV, each unit waits for a unique, specific response time and then sends a ping in response via its own separate transmission channel.

Figure 2 – Buoy-based underwater positioning methods

The standard LBL systems mentioned earlier are not well suited for large groups, as only one AUV can access the buoy network at a time and receive position updates. Therefore, the position update interval increases with the number of vehicles.

New LBL systems like the one shown in Fig. 3(b), synchronized the clocks of the buoys and the AUV transceiver units. The buoys broadcast a ping containing a unique identifier at certain intervals. When the AUV receives this ping, the known beacon broadcast schedule and the timing of the synchronized clocks ensure that the vehicle knows when the ping was sent and can directly calculate the OWTT (one-way travel-time).

Another improvement over conventional LBLs is the system shown in Fig. 3(c). Relying on the setup in Fig. 3(b), the buoys now transmit their GPS positions along with a unique identifier. As with the system described earlier, the AUV does not need to send queries to the buoys. With the buoy positions embedded in the ping, the buoys are free to swim, and there is no need to save their coordinates to the AUV before deployment.

2.2. Slant ranges usage

If there are two or more buoys, it is possible to reconstruct information about the location of the modem-object using the slant range mechanism to recalculate the position in the global coordinate system by GPS or GLONASS.

Two buoys are considered in the task. Their locations at longitudinal, lateral and vertical displacements are determined by the η1(b1)=xb1yb1zb1T and η1(b2)=xb2yb2zb2T vectors. Assuming that the depth is constant (without considering wave perturbations), it is possible to neglect the zb1 and zb2 values, obtaining the positions in the plane pb1=xb1yb1T and pb2=xb2yb2T.

The possibility to obtain slant range to the underwater vehicle will allow to calculate its position, but preliminary requires knowledge of its depth z (assuming that the pressure sensor is always working) to determine the projection of the range on the XY plane. Denoting the slanted ranges as rb1 and rb2, expressions for their plane projections are obtained

rb1=rb12-z2,rb2=rb22-z2.

Further, the positioning problem is reduced to the geometric problem of finding intersection points between two circles whose centers are the buoy positions, and whose radii are the projections found earlier (rb1 and rb2).

The distance between the circles d=pb2-pb1 allows to use the cosine theorem

cosα=rb12+d2-rb222drb1.

The possible positions of the submersible are now defined by the expression

xSRySR=rb1+Udrb1cosα±U⊥rb11-cos2α,

where Ud=pb2-pb1d=UdxUdy – unit vector between the first and the second center,

U⊥=Uy-Ux – perpendicular to the unit vector.

The duality of solving the equation is resolved if the previous underwater vehicle position is known. In this case, it is necessary to choose the point closest to the last vehicle position (Fig. 3).

Figure 3 – Slant ranges geometry

2.3. Underwater vehicle model description

General coordinates of an autonomous underwater vehicle (AUV) are determined in a geocentric coordinate system using SNAME notation [13,14]:

η=η1η2∈R6,#1

where η1=xyzT determines the longitudinal, lateral, and vertical positions, respectively, and the vector η2=φθψT determines the Euler angles: roll, pitch, and yaw, respectively.

The velocity vector ν is expressed in the coordinate system associated with the body. The velocities according to the notation announced above should be written as

ν=ν1ν2=uvwpqrT.#2

Under the influence of hydrodynamic effects caused by the aquatic environment, it is possible to write down the expression for the underwater vehicle dynamics in the following form

Mν+Cνν+Dνν+gη=τ+JT(η)fe,M=MRB+MA,Cν=CRBν+CAν,#3

where MRB∈Rn×n – the solid body inertia matrix (n is the DoF number),

MA∈Rn×n – added mass matrix,

CRBν∈Rn×n – solid body Coriolis matrix,

CAν∈Rn×n – added mass Coriolis matrix,

Dν∈Rn×n – dissipative coefficients matrix,

gη∈Rn×1 – gravitational forces and moments vector,

τ∈Rn×1 – vector (of forces and moments) of the controls applied to the body,

fe∈Rn×1 – vector (of forces and moments) of external disturbances applied to the body.

In real systems, τ is calculated with the thruster’s models using the thruster’s distribution matrix (T) as

τ=Tu,#4

where u∈Rp×1 – is a vector that describes the thrusters' load (p is the number of thrusters), and the matrix T∈Rn×p relates the thrusters' load u and the forces/moments vector τ

The kinematics equation linking (1) and (2) is written in the form

η=J-1ην=RIBη03×303×3Jk,oη-1ν=RBIη03×303×3Jk,o-1ην,#(5)

where RIBη∈R3×3 – the rotation matrix obtained from the Euler angles,

Jk,oη∈R3×3 – the Jacobian linking the angular velocities of the world reference system and the body system.

Jk,oη=10-sinθ0cosφcosθsinφ0-sinφcosθcosφ.

The rotation matrix is defined by the following expression

RIBη=cosψcosθsinψcosθ-sinθ-sinψcosφ+cosψsinθsinφcosψcosφ+sinψsinθsinφsinφcosθsinψsinφ+cosψsinθcosφ-cosψsinφ+sinψsinθcosφcosφcosθ.

Given x=ηηT, it is possible to write the state vector

x=ηM-1(Tu+JT(η)fe-C(ν)ν-D(ν)ν-g(η)).

The latter equation can be represented in the State-Dependent Coefficients (SDC) form [17]

dxdt=Ax,tx+Bx,tu,  x(t0)=x0,

y=Ctx+Dtu,

Ax,tx=06×6J(x)-106×6-M-1Cx+Dx,    Bx,t=06×6M-1,#6

u=Tu+uadd(x), uadd=JT(x)fe-g(x).

where A∈R2n×2n – dynamics matrix,

B∈R2n×n – control matrix,

C∈Rm×2n – output matrix (m – number of system outputs),

D∈Rm×n – input-output coupling matrix,

u∈Rn×1 – control vector, including the vector of external perturbations uadd∈Rn×1.

2.4. Description of the Kalman filter model

Consider the problem of constructing a filter for a nonlinear system, known as an extended Kalman filter. It allows to estimate the controlled object state, even if the dimensionality of the state vector of the system under study exceeds the number of measured parameters. To calculate the current state of an a priori known dynamic system, it is necessary to know its current measured state, as well as the state of the filter at the time of the previous measurement. The purpose of the filter is to minimize the sum of squared errors of the state vector estimation [8,9,10].

Taking into account the white noise of the system w and the white noise of measurements v, the system in SDC-form has the following view:

dxdt=Ax,tx+Bx,tu+Gw,  xt0=x0,y=Ctx+Dtu+Hw+v,#7

The state vector estimation of the Kalman filter is defined as

dxdt=Ax,tx+Bx,tu+KKFty-Ctx,    xt0=x0,KKFt=PtCtTR-1,  Pt0=P0,Pt=Ax,tPt+PtAx,tT+GQGT-PtCtTR-1CtPt,#8

where Pt – is the state covariance estimation, P0 – is the initial value of the state covariance estimation, KKFt – is the Kalman filter gain matrix, Q and R are the cost matrices in the Riccati equation.

Noises w and v must satisfy the conditions:

Ew=Ev=0,  EwwT=Q,  EvvT=R.

3. Navigation system components

3.1. Description of the controlled object MMT-300

The MMT-300 AUV is designed to perform bottom and aquatic surveys at depths of up to 300 m. Search programs (missions) can be described as AUV tacking through the surveyed area with activation of on-board search devices (one or several) at specified intervals and then returning the vehicle to the supplying vessel.

The location of the main AUV elements is shown in Fig. 4: 1 – sustainer propulsion system, 2 – stabilizers, 3 – aft compartment, 4 – charge connector, 5 – aft cover, 6 – navigation and communication compartment, 7 – side-scan sonar, 8 – hermetic payload compartment, 9 – bow cover, 10 – outboard payload compartment, 11 – horizontal thruster, 12 – compartment cover, 13 – radio module, 14, 16 – cargo arm, 15 – autopilot compartment, 17 – vertical thruster, 18, 22 – payload compartment cover, 19 – towing arm, 20 – emergency ballast mechanism, 21 – Doppler velocity logger, 23 – ELS antennas, 24 – hermetic communication connector.

Figure 4 – Location of the AUV’s main components

3.1.1. Software control and on-board navigation system

The Onboard Control and Navigation System (OCNS) is designed to control all systems of the vehicle in all operating modes of the AUV.

The OCNS provides:

− execution of a pre-programmed AUV task;

− detection of emergencies and their adequate handling;

− trajectory control of various types;

− determination of the resulting AUV location in geographical coordinates.

The onboard control and navigation system includes:

− emergency sensors and actuators;

− magnetic compass and orientation sensors;

− radio module;

− depth sensor;

− Doppler logger.

3.1.2. Propulsion-steering complex

The propulsion-steering complex (PSC) is a program-controlled executive device. The AUV uses a propulsion system consisting of four aft reversible thrusters and two thrusters of horizontal and vertical channels. Stern thrusters are located in pairs in the horizontal and vertical planes at an angle of 22 to the longitudinal axis. This propulsion scheme makes it possible to create arbitrary forces and moments to control the AUV, as well as to implement various movement modes. The AUV motion is controlled by five degrees of freedom. The range of possible values for a given longitudinal velocity is 0–2.0 m/s.

3.1.3. Hydroacoustic navigation and communication system (option)

The HNCS provides vehicle tracking, as well as periodic correction in determining the onboard coordinates of the AUV. The error in determining the distance depends significantly on hydrological conditions of the working area, as well as on the error in the set sound velocity. In the absence of GANS, the AUV uses calculus (using magnetic compass (MC), depth sensor (DS) and Doppler velocity logger (DVL) readings) and GNSS (on the surface) to determine its own coordinates.

The main HNCS features are summarized in Table 1.

Table 1 – HNCS features

| Parameter | Value |
| --- | --- |
| Operating depth, m | up to 300 |
| Operating range (slant), km | up to 3.5 |
| Slant range measurement error, m | no more than 0.01 |
| Operating frequency, kHz | 18 … 34 |
| Bearing measurement error, deg | 0.1 |
| Data transfer rate, kbit/s | up to 13.9 |

3.2. MMT-300 mathematical model

The main MMT-300 vehicle parameters are summarized in Table 2.

Table 2 – AUV parameters

| Mass, m | Length, L | Radius, r | Center-of-mass vector, rgb | Buoyancy center vector, rbb | Buoyancy, B |
| --- | --- | --- | --- | --- | --- |
| 150 kg | 3.081 m | 0,147 m | 000T | 000.07T | mg, N |

3.2.1. Thrusters allocation matrix

Given the geometric and physical parameters of the thrusters, it is possible to determine the thrusters allocation matrix (4) by the following expression

ti=firi×fi,  T=t1⋯tn,#9

where fi – is the force created by the i-th thruster,

ri – is the geometric position of the force application point of the i-th thruster relative to the vehicle's center of mass.

It is worth noting that the forces generated by the MMT-300 thrusters depend on the software control, which leads expression (9) to the parametric form

ticode=ficoderi×ficode,  Tcode=t1code⋯tpcode,#10

where code∈-128;128 – is the software control code.

From Fig. 5 it is possible to determine the values ri, which are presented in Table 3.

Figure 5 – MMT-300 geometric parameters

Table 3 – Thrusters moment of arms of the ММТ-300

| Thruster № | ri, (mm) |  |  |
| --- | --- | --- | --- |
|  | lxi, (mm) | lyi, (mm) | lzi, (mm) |
| 1 | -1330 | 0 | 189 |
| 2 | -1330 | 159 | 30 |
| 3 | -1330 | 0 | -129 |
| 4 | -1330 | -159 | 30 |
| 5 | 1330 | 0 | 30 |
| 6 | 882 | 0 | 30 |

Since all thrusters are the same in design, they create a single force that depends on the software control code Fcode. In this case ficode is defined taking into account the geometry of the control object as

f1code=Fcodecos22o0Fcodesin22o, f2code=Fcodecos22oFcodesin22o0,

f3code=Fcodecos22o0-Fcodesin22o, f4code=Fcodecos22o-Fcodesin22o0,

f5code=0Fcode0, f6code=00-Fcode.

From the thruster’s static characteristic (Fig. 6) it is possible to write down the functional dependence between the control code and the generated force, for example by means of a sixth-order polynomial

Fcode=7.2937∙10-13∙code5-6.9155∙10-10∙code4-2.2935∙10-8∙code3++ 2.3559∙10-5∙code2+1.9049∙10-4∙code+0.1332.#11

Figure 6 – Static characteristics of the MMT-300 thrusters

With the dependencies defined above, it is possible to determine Tcode which makes it possible to define the expression for forces and moments (4) already in parametric form τcode=Tcode∙u. In this case, u determines the thruster’s load percentage. At full performance u=111111T.

3.2.2. Inertia and added masses matrices

The matrix MRB в уравнении (6) is constant, symmetric and positively determined (MRB>0). The values filling it largely depend on the geometric configuration and, in the most general case, have the following form [14]

MRB=mI3×3-mSrgbmSrgbIOb∈R6×6,#12

where m – is rigid body mass,

I3×3 – is 3×3 identity matrix,

IOb – is the inertia tensor in the reference frame of the body,

rgb=xgygzgT – is the vector from the origin to the gravity center of the body,

S ∙  – is an operator of vector transformation into a skewed matrix:

Sx∈R3×1=0-x3x2x30-x1-x2x10.#13

MRB as expanded:

MRB=m000mzg-myg0m0-mzg0mxg00mmyg-mxg00-mzgmygIxx-Ixy-Ixzmzg0-mxg-IyxIyy-Iyz-mygmxg0-Izx-IxyIzz.

The inertia tensor for a solid cylinder is defined as

IOb=12mr2000112m3r2+L2000112m3r2+L2.#14

The matrix MA consists of the external forces and moments elements, and is written in composite form as [13]

MA=-XuXvXwXpXqXrYuYvYwYpYqYrZuZvZwZpZqZrKuKvKwKpKqKrMuMvMwMpMqMrNuNvNwNpNqNr,

where Xv=∂X∂v, Yu=∂Y∂u, etc.

Given the symmetric form approximation of the underwater vehicle, the matrix notation will be greatly simplified and will take a diagonal form:

MA=-diagXu, Yv, Zw, Kp, Mq, Nr=MATr∈R3×3⋱⋱MARot∈R3×3#15

There is no unique correct way to calculate the MA matrix elements, so, as a rule, a variety of estimation methods are used. For a solid cylinder of mass m, length L, radius r the added masses (15) are defined as [13]

Xu=-0.1m,Yv=-πρr2L,Zw=-πρr2L,Kp=0,Mq=-112πρr2L3,Nr=-112πρr2L3.#16

3.2.3. Coriolis matrix

According to Fossen [14], the matrix Cν is generally defined from the blocks of the inertia matrix M. In this case M=M11M12M21M22, then

Cν=03×3-SM11ν1+M12ν2-SM11ν1+M12ν2-SM21ν1+M22ν2∈R6×6.#17

3.2.4. Gravitational and buoyancy vector

In [14], to calculate gη, the author uses the expression

gη=mg-Bsinθ-mg-Bcosθsinϕ-mg-Bcosθcosϕ-ygmg-ybBcosθcosϕ+zgmg-zbBcosθsinϕzgmg-zbBsinθ+xgmg-xbBcosθcosϕ-xgmg-xbBcosθsinϕ-ygmg-ybBsinθ,#18

where m – is the vessel's mass including water in space,

g – is the acceleration of gravity,

B=ρg∇ – is the buoyancy force,

where ρ – is the liquid density,

∇ – is the liquid volume displaced by the vessel,

xb, yb, zb – are the components of the vector rbb=xbybzbT from the origin to the center of buoyancy.

3.2.5. Damping forces

FL (lift)

FD (drag)

Figure 7 – Damping force distribution diagram

The drag force/lifting force is calculated with

FD=CDρv22SD,  FL=CLρv22SL,

where CD – is the drag coefficient,

CL – is the lift coefficient,

ρ – is the environmental density,

v – is the body velocity in the fluid,

SD – is the surface area, relative to the flow,

SL – is the surface area perpendicular to the flow,

The CL coefficients for the cylindrical approximation form AUV are given in [15]. For a given ratio of length and radius, the corresponding coefficient is CL=0.64.

The CD coefficient is determined relative to the bow cover’s geometry. In this case, the closest shape is a hemispherical bowl. In [15] the corresponding value of CD=0.4.

The frontal and side areas are determined solely from the basic geometric parameters of the submersible by the following expressions:

SD=π0,1472=0.06789 (m2).

SL=2π∙0,147∙3,081=2.8457 (m2).

The forces induced by the cylindrical body motion FDAMP=FDOXFLOYFLOZT, are approximately defined as

FDOX=0.41000∙u220.06789=13.578u2,

FLOY=0.641000∙v222.8457=910.624v2,

FLOZ=0.641000∙w222.8457=910.624w2,

The moments created by the cylindrical body motion are approximately defined as

MDAMP=ν1×MATrν1.

The damping forces, taking into account the calculated parameters are defined as

Dνν=diagFDAMP,MDAMP.

3.3. Mathematical model of the buoy

The scheme of the buoys involved in the system is shown in Fig. 8. Their parameters are summarized in Table 4.

Figure 8 – The schematic representation of the buoy in its reference frame

Table 4 – Buoy parameters

| Mass, m | Length, L | Radius, r | Center-of-mass vector, rgb | Buoyancy center vector, rbb | Buoyancy, B |
| --- | --- | --- | --- | --- | --- |
| 12 kg | 0.36 m | 0.08 m | 00-0.16T | 000.21T | 171, N |

Given that the buoy has a cylindrical shape, it is possible to describe its dynamics similar to MMT-300, except that the inertia tensor is rotated and defined as

IOb=112m3r2+L2000112m3r2+L200012mr2.

3.4. Description of on-board sensors

No one sensor can measure perfectly, i.e., without errors. An error is the difference between the true value and the actual measured value. Sensor errors are divided into random and systematic components. Random components are also called random measurement errors. Their specific realization at a particular time cannot be predicted; it is only possible to describe a general pattern of their behavior. The mathematical model of random components is defined as a sequence of the white noise counts sum with Markov process counts or flicker noise [24]. Determining the characteristics of the random processes allows us to correctly describe their dynamics when integrating the IMU with other sensors, improving the filtering quality.

3.4.1. Inertial sensors

When combining the IMU with other sensors, the zeropoint bias of accelerometers and gyroscopes are included in the vector of estimated parameters: this allows to take into account their random component, which is the main source of positioning errors after compensating for other systematic errors. Temperature has the greatest effect on the zero-offset of MEMS-sensors; at the same time, its influence on the axis unorthogonality and scale coefficient errors is limited.

The output vector of velocity measurements from the 3-axis accelerometer unit is modeled as

νIMU=ν+wIMU+bIMU,

where wIMU~N0,σIMU2, σIMU2 – RMS error of the accelerometer, bGYRO – is acceleration bias, which is modeled as a 1st-order Markov process.

Similarly, the real output of the gyroscope is defined as

η2GYRO=η2+wGYRO+bGYRO,

where wGYRO~N0,σGYRO2, σGYRO2 – RMS error of the accelerometer, bGYRO – is gyroscope bias, which is modeled as a 1st-order Markov process.

3.4.2. Depthmeter (pressure sensor)

Depth and underwater pressure have a direct correlation. As the unit goes deeper into the water, the pressure readings increase linearly.

The actual depth sensor output is simulated by adding noise to the actual depth

z=z+wz,

where wz~N0,σz2, σz2 – RMS error in depth.

3.4.3. Doppler velocity logger

The Doppler logger measures the change in acoustic frequency to determine the vehicle's speed relative to the seafloor. The actual Doppler logger output is modeled by adding noise to the actual vehicle speed

uDVLvDVL=uv+wuwv,

where wu~N0,σDVL2, wv~N0,σDVL2, σDVL2 – RMS of the velocity error.

3.4.4. Underwater acoustic positing system

The underwater acoustic positing system measures the distance and direction of the vehicle from the reference positions. It can be interfaced by GPS to provide Earth-related coordinates. However, acoustic position estimate is effected by GPS accuracy, system installation, ship attitude, sound velocity profile, ray bending, and measurement noise [18].

Assuming the system is precisely calibrated, buoys installation and attitude have negligible effects. The mathematical model of the system actual output is given as

pUAPS=pUAPS+wUAPS+bUAPS,

where wUAPS~N0,σUAPS2, σUAPS2 – RMS positional error, bGYRO – is the time-varying bias modeled as a 1st-order Markov process and depends on the sound velocity profile and ray bending effect.

3.5. Sensors fusion

According to Fig. 1, it is required to combine information from the sensors to form a measurements vector for subsequent filtering. There are numerous methods of combining sensor readings.

In telecommunications, maximum ratio combining (MRC) is a data fusion method in which: the signals of each channel are summed, the gain of each channel is set proportional to the RMS signal level and inversely proportional to the RMS noise level in that channel:

sopt=σsy2sx+σsx2syσsx2+σsy2.

Maximum ratio combining is the optimal fusion tool for independent channels with additive white Gaussian noise. If hydroacoustic buoys unambiguously determine the position, it is necessary to combine the Doppler logger data with the IMU in the measurements vector:

u=σDVL2uIMU+σIMU2uDVLσIMU2+σDVL2, v=σDVL2vIMU+σIMU2vDVLσIMU2+σDVL2.

4. Navigation system modeling

The filtering software simulation considers the following situation: the MMT-300 moves to a predetermined point, and two hydroacoustic buoys are located on the water surface. The influence of surface current is taken into account by influencing the displacement of buoys. If the measurement error is larger than the state uncertainty estimate, the filter will "trust" the simulation data more. That is why it is important to correctly select the values of covariance matrices, the main tool for tuning the filter. With the given measurement devices noise parameters, the EKF parameters are as follows:

P0=I12×12, Q=0,

x0=x0+wx0~N0,0.05,

R=diag0.8;0.8;0.102;0.15;0.15;0.15;0.25;0.25;0.35;0.14;0.14;0.14.

It is assumed that a Doppler logger is used when fusing sensor information due to the small distance from the seafloor. It should also be noted that zero-order extrapolators with update frequencies, presented in the table below, are used to simulate the discrete nature of the measurement devices.

Table 5 – Update rates of the sensors

| Sensor | Update rate |
| --- | --- |
| IMU | 50 Hz |
| GPS | 1 Hz |
| DVL | 25 Hz |
| PS | 50 Hz |

The block diagram of the system model is shown in Fig. 9 and reflects its main components: hydroacoustic buoys to obtain slant ranges to AUV, controller to send control signals, dynamics/kinematics block of the underwater vehicle and sensors fusion block.

dxdt=Ax,tx+Bx,tu+Gw,  y=Ctx+Dtu+Hw+v,

Buoy 1

Buoy 2

Sensors Fusion

dxdt=Ax,tx+Bx,tu+KKFty-Ctx

Controller

y

rb2

rb1

y

u

x

Extended Kalman filter

Dynamics and kinematics of the ММТ-300

Figure 9 – Positioning system block diagram

The simulation results are presented below. The initial state of the underwater vehicle in this case is defined as zero. Fig. 10 shows the trajectories of the submersible and the hydroacoustic buoys.

Figure 10 – XY position of buoys and AUV

The charts of the system output are shown in Fig. 11. The measurement sensor errors are evident here: noise and high update frequencies are noticeable. The results of the Kalman filter for state recovery are shown in Fig. 12 and show more appropriate results relative to the actual data.

Figure 11 – Measurement charts

A more correct assessment of the filter performance can be made by the error with respect to the real data (Fig. 13). Quantitative evaluation is already defined by quality metrics. MSE, MAE and RMSE metrics are summarized in Table 6.

Figure 12 – EKF estimation charts

Figure 13 – EKF estimation error charts

Table 6 – Quality metrics

| Metric | Value |
| --- | --- |
| MSE (η) | 1.8683273.2919730.3908710.0302600.0442630.318385T |
| MSE (ν) | 0.1917050.1154270.0632610.0024180.0434860.013741T |
| MAE (η) | 0.0299130.0824230.0083490.0012440.0020440.004935T |
| MAE (ν) | 0.0032270.0016750.0046790.0000300.0002470.000525T |
| RMSE (η) | 0.0745810.1167120.1395430.0007320.0039810.016776T |
| RMSE (ν) | 0.0072490.0041720.0023440.0000380.0003890.000886T |

5. Conclusion

The main goal of this paper was to develop a high-quality autonomous navigation performance using MEMS sensor-based IMUs with a rough accuracy class. During this study, a simulation model of underwater robot motion was developed. All proposed algorithms and approaches were tested with simulation modeling tools and using natural data.

The specified goal was achieved: it was shown that it is practically possible to achieve acceptable quality of navigation in autonomous mode using an IMU of rough accuracy class. The positioning error turns out to be limited over a long-time interval even if the motion starts from a large area of initial uncertainty.

Further research will be focused on the generation of control laws for a swarm of submersibles relative to the leader, since the main problem of cooperative navigation and positioning still remains state estimation, and accurate collection of observations, especially for the leading element.

References

- Miller, A., Miller, B., Miller, G., 2021. Navigation of Underwater Drones and Integration of Acoustic Sensing with Onboard Inertial Navigation System. Drones 5, 83. https://doi.org/10.3390/drones5030083.

- Li, J., Gu, M., Zhu, T., Wang, Z., Zhang, Z., Han, G., 2023. Research on Error Correction Technology in Underwater SINS/DVL Integrated Positioning and Navigation. Sensors 23, 4700. https://doi.org/10.3390/s23104700.

- Otero, P., Hernández-Romero, Á., Luque-Nieto, M.-Á., Ariza, A., 2023. Underwater Positioning System Based on Drifting Buoys and Acoustic Modems. JMSE 11, 682. https://doi.org/10.3390/jmse11040682.

- Li, P., Liu, Y., Yan, T., Yang, S., Li, R., 2023. A Robust INS/USBL/DVL Integrated Navigation Algorithm Using Graph Optimization. Sensors 23, 916. https://doi.org/10.3390/s23020916.

- González-García, J., Gómez-Espinosa, A., Cuan-Urquizo, E., García-Valdovinos, L.G., Salgado-Jiménez, T., Cabello, J.A.E., 2020. Autonomous Underwater Vehicles: Localization, Navigation, and Communication for Collaborative Missions. Applied Sciences 10, 1256. https://doi.org/10.3390/app10041256.

- Kebkal, K.G., Mashoshin, A.I., 2017. AUV acoustic positioning methods. Gyroscopy Navig. 8, 80–89. https://doi.org/10.1134/S2075108717010059.

- Pacini, F., Paoli, G., Kebkal, O., Kebkal, V., Kebkal, K., Bastot, J., Monteiro, C., Sucasas, V., Schipperijn, B., 2018. Integrated comunication network for underwater applications: the SWARMs approach, in: 2018 Fourth Underwater Communications and Networking Conference (UComms). Presented at the 2018 Fourth Underwater Communications and Networking Conference (UComms), IEEE, Lerici, pp. 1–5. https://doi.org/10.1109/UComms.2018.8493214.

- Zhao, L., Dai, H.-Y., Lang, L., Zhang, M., 2022. An Adaptive Filtering Method for Cooperative Localization in Leader–Follower AUVs. Sensors 22, 5016. https://doi.org/10.3390/s22135016.

- Zhang, F., Wu, X., Ma, P., 2022. Consistent Extended Kalman Filter-Based Cooperative Localization of Multiple Autonomous Underwater Vehicles. Sensors 22, 4563. https://doi.org/10.3390/s22124563.

- Sheng, G., Liu, X., Sheng, Y., Cheng, X., Luo, H., 2023. Cooperative Navigation Algorithm of Extended Kalman Filter Based on Combined Observation for AUVs. Remote Sensing 15, 533. https://doi.org/10.3390/rs15020533.

- Almeida, J., Matias, B., Ferreira, A., Almeida, C., Martins, A., Silva, E., 2020. Underwater Localization System Combining iUSBL with Dynamic SBL in ¡VAMOS! Trials. Sensors 20, 4710. https://doi.org/10.3390/s20174710.

- Huang, J., 2023. An Underwater Target Tracking Algorithm Based on Extended Kalman Filter. Mobile Information Systems 2023, 1–12. https://doi.org/10.1155/2023/9916531.

- Antonelli, G., 2014. Underwater Robots, Springer Tracts in Advanced Robotics. Springer International Publishing, Cham. https://doi.org/10.1007/978-3-319-02877-4.

- Fossen, T.I., 2021. Handbook of marine craft hydrodynamics and motion control =: Vademecum de navium motu contra aquas et de motu gubernando, Second edition. ed. Wiley, Hoboken, NJ.

- Blevins, R.D., 2003. Applied fluid dynamics handbook, Reprint ed. with corrections. ed. Krieger Pub, Malabar, Fla.

- https://ntrs.nasa.gov/citations/19930091586 (accessed 25.06.2023).

- Kabanov, A., Kramar, V., Lipko, I., Dementiev, K., 2022. Cooperative Control of Underwater Vehicle–Manipulator Systems Based on the SDC Method. Sensors 22, 5038. https://doi.org/10.3390/s22135038.

- Shaukat, N., Ali, A., Javed Iqbal, M., Moinuddin, M., Otero, P., 2021. Multi-Sensor Fusion for Underwater Vehicle Localization by Augmentation of RBF Neural Network and Error-State Kalman Filter. Sensors 21, 1149. https://doi.org/10.3390/s21041149.

- Kabanov, A., Kramar, V., 2022. Marine Internet of Things Platforms for Interoperability of Marine Robotic Agents: An Overview of Concepts and Architectures. JMSE 10, 1279. https://doi.org/10.3390/jmse10091279.

- Luo, Q., Yan, X., Wang, Chenxu, Shao, Y., Zhou, Z., Li, J., Hu, C., Wang, Chuntao, Ding, J., 2021. A SINS/DVL/USBL Integrated Navigation and Positioning IoT System with Multiple Sources Fusion and Federated Kalman Filter (preprint). In Review. https://doi.org/10.21203/rs.3.rs-1111330/v1.

- Morvan, P.-Y., Bagot, G., 2021. Innovative software solutions for subsea positionings (other). pico. https://doi.org/10.5194/egusphere-egu21-7984.

- Caiti, A., Di Corato, F., Fenucci, D., Allotta, B., Costanzi, R., Monni, N., Pugi, L., Ridolfi, A., 2014. Experimental results with a mixed USBL/LBL system for AUV navigation, in: 2014 Underwater Communications and Networking (UComms). Presented at the 2014 Underwater Communications and Networking (UComms), IEEE, Sestri Levante, Italy, pp. 1–4. https://doi.org/10.1109/UComms.2014.7017129.

- Kebkal, K.G., Kebkal, O.G., Yakovlev, S.G., Bannasch, R., 2012. Experimental Performance of a Hydro-Acoustic USBL-Aided LBL Positioning and Communication System. IFAC Proceedings Volumes 45, 249–254. https://doi.org/10.3182/20120410-3-PT-4028.00041.

- Quinchia, A., Falco, G., Falletti, E., Dovis, F., Ferrer, C., 2013. A Comparison between Different Error Modeling of MEMS Applied to GPS/INS Integrated Systems. Sensors 13, 9549–9588. https://doi.org/10.3390/s130809549.